European Maritime Single Window environment

Common Location Database

Technical specifications, standards and procedures

Version: 0.3

Date: 07/04/2020

Document History

| Version | Date | Changes | Prepared | Approved |
| --- | --- | --- | --- | --- |
| 0.1 | 02/03/2020 | Draft | EMSA |  |
| 0.2 | 20/03/2020 | Comments from COM | EMSA |  |
| 0.3 | 07/04/2020 | Review with COM on management principles | EMSA |  |

Document information

| Creation file | 07/04/2020 |
| --- | --- |
| Filename | European Maritime Single Window environment Common Location Database - Technical specifications, standards and procedures |
| Location | … |
| Number of pages | 21 |

Table of Contents

1.	Introduction	5

2.	Objective of the document	5

3.	EMSWe Common Location Database overview	5

3.1	UN/LOCODE	6

3.2	Port facilities	6

3.3	SSN-Specific codes	7

3.4	Dataset of the EMSWe CLD	7

4.	Management of the EMSWe CLD	7

4.1	Roles and responsibilities	7

4.1.1	Member States – Maritime National Single Windows	7

4.1.2	The Commission – EMSWe CLD	8

4.2	Principles of management	8

5.	Interfaces with Maritime National Single Windows	9

5.1	System to System (S2S) mechanisms	9

5.1.1	Request/response	10

5.1.2	Announcement (“push”)	10

5.2	Web interface	11

5.3	Access Rights	11

6.	Operational services and procedures	11

6.1	Overview	11

6.2	Procedures for updating the EMSWe CLD	12

6.2.1	UN/LOCODE	12

6.2.1.1	Updating UN/LOCODE list	12

6.2.1.2	Synchronising EMSWe CLD with UN/LOCODE	12

6.2.2	Port facilities	13

6.2.2.1	Updating port facilities in IMO GISIS	13

6.2.2.2	Synchronising the EMSWe CLD with IMO GISIS	13

6.2.3	SSN specific codes	13

6.2.3.1	Updating SSN specific codes	13

6.2.3.2	Synchronising EMSWe CLD with SSN specific codes list	13

7.	Commissioning measures for establishing connection with MNSWs	13

7.1	General guidance	13

7.2	Commissioning Procedure	14

8.	System performance	14

8.1	Timeframes for Data Availability	14

8.2	Timeframes for Data Storage	14

8.3	System Availability Requirements	14

8.4	Backup procedures	15

8.5	Continuity of Service	15

8.6	Additional System Performance Requirements	15

8.7	Operational Coordination	15

9.	System security	15

9.1	General provisions	15

9.2	Security measures	15

9.2.1	Access Control	15

9.2.2	Authentication	16

9.2.3	Authorisation	16

9.2.4	Traceability and accountability	16

9.2.5	Confidentiality	16

9.2.6	Integrity	17

9.2.7	Training	17

9.2.8	Audit	17

Annex 1	Data elements of the EMSWe CLD	18

List of Figures

Figure 1: Types of accesses offered by the EMSWe CLD	10

List of Abbreviations

| CLD | Central Location Database |
| --- | --- |
| EMSA | European Maritime Safety Agency |
| EMSWe | European Maritime Single Window environment |
| EUROSTAT | The statistical office of the European Union |
| GISIS | IMO Global Integrated Shipping Information System |
| HLSG | High-Level Steering Group for Governance of the Digital Maritime System and Services |
| IMO | International Maritime Organization |
| ISO | International Organization for Standardization |
| ISPS | International Ship and Port Facility Security Code |
| MNSW | Maritime National Single Window |
| MS | Member State |
| NCA | Used to identify a SSN National Competent Authority |
| SIG | System Interface Guide |
| SOAP | Simple Object Access Protocol |
| SSN | SafeSeaNet |
| UN/LOCODE | United Nations Code for Trade and Transport Locations |
| UNECE | The United Nations Economic Commission for Europe (UNECE) |
| UPU | Universal Postal Union |

# Introduction

Article 15 of the EMSWe Regulation (EU) 2019/1239 foresees that a Common Location Database (CLD) shall be established and be made available to the Maritime National Single Windows (MNSW) for facilitation of ship reporting.

The Commission shall adopt implementing acts laying down the technical specifications, standards and procedures for the setting up of the EMSWe CLD with respect to collecting, storing, updating and provision of the location and port facility codes.

At its 6th meeting on 20 January 2020, the HLSG agreed to entrust the SSN Group with specific tasks in relation to the provision of technical advice and expertise on the substance of the Implementing Regulations that the Commission intends to adopt in respect of provisions in articles 14 (EMSWe Ship Database), 15 (Common Location Database) and 16 (Common Hazmat Database) of the EMSWe Regulation.

According to the mandate given to the SSN Group for the EMSWe Databases, the group will carry out the following tasks:

- Definition of the dataset of the EMSWe CLD;

- Analysis of processes related to collecting, storing, updating and provision of information to MNSWs;

- Elaboration of functional and non-functional requirements;

- Definition of roles and responsibilities of parties involved;

- Definition of system interfaces between the common databases and MNSWs;

- Definition of commissioning measures for establishing connection with MNSWs.

The work done by the SSN subgroup on the EMSWe CLD will be reported to the EMSWe expert subgroup for further consultation. The SSN group will also report to the HLSG on the work done. The draft Implementing Regulations, new or amended, will be submitted by MOVE.D1 to the Digital Transport and Trade Facilitation Committee for opinion before being adopted by the Commission.

# Objective of the document

The objective of this document is to specify the technical specifications, standards and procedures for the EMSWe CLD as indicated in the mandate of the SSN Group for the EMSWe Databases.

This document contains the high level technical, functional and operational requirements of the EMSWe CLD, while more detailed specifications will be described in technical and operational documentation such as the System Interface Guides and the User Manual. Such technical and operational documentation will be referred to in the implementing act laying down the technical specifications, standards and procedures for the EMSWe CLD but will be kept as distinct documents in order to allow faster updates and corrections.

This is a working document of the SSN Group. The proposals that need to be approved by the Group are included in the document with a specific formatting: “Proposal no [number]”.

# EMSWe Common Location Database overview

In the whereas (point 23) of the EMSWe Regulation, it is specified that the EMSWe CLD shall hold a reference list of location codes, including the United Nations Code for Trade and Transport Locations (UN/LOCODE), the SafeSeaNet-specific codes, and the port facility codes as registered in the Global Integrated Shipping Information System (GISIS) of the International Maritime Organization (IMO).

Article 15 of the same Regulation does not refer to the SafeSeaNet-specific codes when defining content of the EMSWe CLD.

Proposal no 1: Considering the role and usage of the SSN-specific codes which address locations which are not covered by the UN/LOCODE but are nevertheless generally used by ports and EU institutions (e.g. EUROSTAT), it is proposed to include them under the scope of the EMSWe CLD.

The below subsections describe all types of codes to be registered in the EMSWe CLD.

## UN/LOCODE

The "United Nations Code for Trade and Transport Locations" is commonly more known as "UN/LOCODE". Although managed and maintained by the UNECE, it is the product of a wide collaboration in the framework of the joint trade facilitation effort undertaken within the United Nations.

UN/LOCODE is used by most major shipping companies, by freight forwarders and in the manufacturing industry around the world. It is also applied by national governments and in trade related activities, such as customs, statistics where it is used by the European Union, by the UPU for certain postal services, etc.

A five-character code element is provided for each location included in the UN/LOCODE list and consists of:

a)	two letters identifying the country, according to the ISO 3166-1 two-letter Code for the representation of names of countries;

b)	followed by three letters identifying the location within the country. Where all permutations have been exhausted, numerals from 2 to 9 can also be used.

For example, the port of Le Havre in France is codified as FRLEH (the first two letters FR identify the country and the following three the code of the port).

UN/LOCODE is managed, maintained and updated by the UNECE secretariat. The code is held in a relational database and presented on a dedicated web-site. UN/LOCODE is freely available to all interested users. It can be consulted and downloaded from the web-site www.unece.org/cefact/locode.

The UN/LOCODE Entry Request System enables registered users to submit request for new UN/LOCODE entries or for certain changes in existing UN/LOCODE entries.

The UNECE Secretariat publishes a new UN/LOCODE version twice a year (July and December). The cut-off date for providing updates to UNECE is 30 April for July updates and 31 October for December updates.

The most updated version is published at: http://www.unece.org/cefact/codesfortrade/codes_index.html

## Port facilities

As defined in the SOLAS Convention (Chapter XI-2 Regulation 1), a port facility is a location, as determined by the Contracting Government or by the Designated Authority, where a ship/port interface takes place. This includes areas such as anchorages, waiting berths and approaches from seaward, as appropriate.

The locations covered by the approved port facility security plans shall be registered in the Maritime Security Module of the Global Integrated Shipping Information System (GISIS) which is maintained by the IMO. This can be done by the Contracting Governments using web interface or via SOAP based web services.

Each port facility is identified by an “IMO Port Facility Number.” It consists of a five-character LOCODE corresponding to a port and a 4-digit code separated with a dash. For example, the port facility “Baltic General Cargo Terminal” at the port of Gdynia in Poland is identified as PLGDY-0004.

The list of port facilities is available to registered users in the Maritime Security Module of the Global Integrated Shipping Information System (GISIS). Access to GISIS requires an account which may be obtained online at no cost: https://webaccounts.imo.org/Common/WebLogin.aspx?ReturnUrl=%2fDefault.aspx .

## SSN-Specific codes

When operating SafeSeaNet system it was found that UN/LOCODE list does not cover all operational cases in maritime domain and it was required to create a specific group of codes for the following cases:

- Unknown port of destination

It is a common practice in the shipping industry for vessels to leave port without knowing their exact port of destination. For example, a vessel may leave a port for an area of destination with way points like the Strait of Gibraltar, the North Sea, the Suez Canal, etc., or for ports outside of the EU where no LOCODEs have yet been specified on the UN/LOCODE list. Waypoints used to define intermediate locations (areas) on a planned vessel’s route when the next port is unknown can be found in Annex 1. The LOCODE “ZZUKN” was created to identify unknown port of destination.

- Call cancellation

Changes to ship voyage plan requires frequently cancellation of previously foreseen port calls. The LOCODE “ZZCAN” was created to be used when it is necessary to cancel a Pre-arrival notification.

- Creation of SSN specific code when updating of UN/LOCODE list is in progress

As a temporary solution, SSN specific code can be created while the process of creating/updating a LOCODE in the UN/LOCODE list is in progress.

- Creation of SSN specific codes for non-EU ports

Non-EU LOCODEs confirmed by a reliable source as having a port function can be created as SSN specific code.

- Inclusion of EUROSTAT codes in SSN system

SSN group on its 21st meeting agreed to add to the SSN specific codes the following EUROSTAT codes:

- EUROSTAT unknown port code, when the country is known but the specific port is unknown. The format is to indicate the two letters identifying the country (according to the ISO 3166 two-letter Code) plus the “888” (e.g. US888, country of destination United States unknown port);

- EUROSTAT code for off-shore installations, when the country is known but the specific off-shore installation is unknown. The format is to indicate the two letters identifying the country (according to the ISO 3166 two-letter Code) plus the “88P” (e.g. DK88P for off-shore installations in Danish waters);

- EUROSTAT code for ship-to-ship transfer, when the country is known but the exact location of the operation is unknown. The format is to indicate the two letters identifying the country (according to the ISO 3166 two-letter Code) plus the “88R” (e.g. DK88R for ship-to-ship transfer in Danish waters).

## Dataset of the EMSWe CLD

As defined in the Article 2 of the EMSWe Regulation, a ‘data element’ means the smallest unit of information which has a unique definition and precise technical characteristics such as format, length and character type.

The detailed description of all data elements of the EMSWe CLD can be found in Annex 1.

# Management of the EMSWe CLD

## Roles and responsibilities

### Member States – Maritime National Single Windows

Member States shall establish a link between MNSWs and the EMSWe CLD and maintain it to ensure that information from the location database is available at national level through the MNSW.

The national coordinator for the EMSWe shall be responsible for the management of the MNSW, which shall include the national coordination of data users as well as ensuring that the necessary national IT infrastructure and the procedures described in this document are established and maintained.

Member States shall maintain a local copy of the information from the EMSWe CLD and make it available at national level to the users of the maritime National Single Window.

The national coordinator for the EMSWe shall develop and maintain the necessary interfaces for automatic transmission of data by electronic means to ensure that the local copy is always synchronized with the EMSWe CLD.

As defined in Directive 2002/59/EC, the SSN NCA shall ensure that UN/LOCODES are designated. Member States shall coordinate the creation/deletion/updating of UN/LOCODEs with the National Focal Points that are officially designated to manage UN/LOCODEs. The list of UN/LOCODE National Focal Points is available at:  http://www.unece.org/cefact/locode/focalpoint.html

As defined in the SOLAS Convention (Chapter XI-2 Regulation 13), Contracting Governments shall communicate to IMO the locations within their territory covered by the approved port facility security plans and thereafter update such information as and when changes relating thereto occur. Member States shall coordinate the creation/deletion/updating of port facilities with the national authorities responsible for port facility security.

### The Commission – EMSWe CLD

The Commission is responsible for the establishment and the availability of the EMSWe CLD and for the oversight of this database in cooperation with Member States.

The Commission/EMSA, in cooperation with the Member States is responsible for:

- the technical implementation and documentation of the EMSWe CLD;

- development, operation and integration of the electronic messages and data;

- maintenance of the interfaces with the EMSWe CLD.

The Commission/EMSA shall ensure the availability of the EMSWe CLD to the MNSWs in order to facilitate ship reporting. The interfaces available for the MNSWs are described in section 5.

The Commission/EMSA shall ensure that the information available in the EMSWe CLD is up to date and corresponds to data stored in UN/LOCODE list (UN/LOCODE), IMO GISIS (port facilities) and SSN system (SSN specific codes).

The Commission/EMSA shall ensure that all updates (i.e. changes to location codes) in the EMSWe CLD are done in line with the procedures described in section 6.

The Commission/EMSA will appoint a EMSWe CLD Administrator (within EMSA or within the Commission) who will be responsible for:

- user administration (setup and maintaining of accounts for human and system users);

- ensuring that the information available in the EMSWe CLD is up to date and corresponds to data stored in UN/LOCODE list (UN/LOCODE), IMO GISIS (port facilities) and SSN system (SSN specific codes);

- coordinating execution of the Commissioning Tests;

- ensuring that the operational and technical documentation is up to date and available to Member States;

- collecting feedback from national coordinators for the EMSWe on the EMSWe CLD database and preparing necessary changes to this database.

## Principles of management

Proposal no 2: The mandate of the EMSWe subgroup once the EMSWe CLD is operational, and where necessary the high-level steering group, will be extended to:

- make recommendations to improve the effectiveness and security of the EMSWe CLD,

- provide appropriate guidance for the development of the EMSWe CLD;

- assist the Commission in reviewing the performance of the EMSWe CLD;

- approve this document, and any amendments thereto;

- liaise with other relevant working forums.

# Interfaces with Maritime National Single Windows

According to point 2 of Article 15 of the EU Regulation 2019/1239 Commission shall ensure the availability of the location database to the maritime National Single Windows in order to facilitate ship reporting.

One connection per Member State will be established between the EMSWe CLD and MNSW. Member States shall maintain a local copy of the information from the EMSWe CLD and make it available to all parties requiring this information, such as declarants, port community systems and other authorities. The EMSWe CLD shall use industry standards and be able to create, update or receive information.

The CLD information will be available to Member States via the following services:

Access through System to System interface

- Announcement (“push”): this service will be used by the EMSWe CLD to announce to MNSW system which previously subscribed to the service the creation, update or deletion of Location records;

- Request/response mechanism: this service will be used by MNSWs to request the content of location records in the EMSWe CLD based on different criteria (e.g. country of the Location).

Access through the web interface

- Web interface: the web interface will be accessible to authorised users through the EMSA portal.

Figure 1 illustrates the interfaces (more explanations about the interfaces are presented in paragraph 5.1 and 5.2:

Figure 1: Types of accesses offered by the EMSWe CLD

## System to System (S2S) mechanisms

These interfaces will be available for exchanging data sets and information in an automated way using a set of defined formats. The message-based interface will support the request, response and distribution (“push”) services. This document contains short description of available mechanism and the technical aspects related to S2S interface will be specified in the EMSWe CLD System Interface Guide (SIG).

### Request/response

The Request/Response mechanism will allow the MNSW to request the content of location records in the EMSWe CLD based on different criteria (e.g. country of the location, name of the location, location code, updates in the specific period, etc.).

The following figure outlines the expected synchronous flow of XML messages related to this transaction:

| MNSW |  | EMSWe CLD |
| --- | --- | --- |
| MS2SSN_Location_Req |  | HTTP Status Code OK SSN2MS_Locatio_Res |

The MS2SSN_Location_Req message is sent by MNSW to request the details registered in the EMSWe CLD for a specific location or a list of locations. The SSN2MS_Location_Res message is the response sent by the EMSWe CLD to MNSW which has requested location details.

Proposal no 3: It is proposed that this mechanism is only used to create an initial local copy of the EMSWe CLD information in the MNSWs.

### Announcement (“push”)

This service will be used by the EMSWe CLD to announce to the MNSWs which have previously subscribed to the service, the latest details of locations at each modification (update or deletion) or creation of a location.

The following figure outlines the expected two synchronous flows of XML messages (announcement requests and the announcements responses) related to this transaction:

| MNSW |  | EMSWe CLD |
| --- | --- | --- |
| MS2SSN_LocationAnn_Req HTTP Status Code 2xx |  | HTTP Status Code OK SSN_Receipt SSN2MS_LocationAnn_Res |

The MS2SSN_LocationAnn_Req message is sent by MNSW to EMSWe CLD for subscription to the Location data announcement (data “push”) service or cancelation of an existing subscription. The message contains information to enable the EMSWe CLD to configure the service.

The SSN2MS_LocationAnn_Res message is sent by the EMSWe CLD to a subscriber. The message contains all the information registered in the most recent version (the one created just before the dispatch of the announcement message) of the location records in the EMSWe CLD.

Proposal no 4: It is proposed to investigate whether the “push” mechanism could provide automatically the full list of location codes after subscribing to the service (i.e. initial synchronisation of databases).

## Web interface

The web interface will be accessible to authorised users (login and password required). This interface will be available for:

- consulting information (search based on different criteria such as location code, country, type of code, etc.);

- creating or updating information (the EMSWe CLD administrator will be able to create or update information about location code in line with the agreed procedures);

- downloading list of location codes based on different criteria (it will be possible to download list of UN/LOCODEs, port facilities, SSN specific codes in XLS format);

- administrating the database (functionalities to ensure synchronisation with other databases, logs, configuration of connections with MNSW).

The web interface may be used as a back-up solution to the S2S interface to retrieve information and update the location database of the MNSW.

Proposal no 5: It is proposed to establish a dedicated web interface for the EMSWe CLD with the EMSWe relevant data elements. This web interface will be distinct from the SSN Central Location Database.

## Access Rights

Access to the EMSWe CLD is controlled via a formal user registration process. Each user is identified by a unique user ID, so that they can be monitored and held accountable for their actions.

The EMSWe CLD administrator will have the following permissions:

- Consultation of information about location records via web interface,

- Download of lists with location codes via web interface,

- Creation/Update/Removal of location records via web interface,

- Logs monitoring via web interface,

- Creation of system and human users.

The national coordinators for the EMSWe will have the following permissions:

- Consultation of information about location records via web interface,

- Download of lists with location records via web interface,

- Configuration of the system to system interface between the EMSWe CLD and the MNSW.

The management of users’ access to location data within the MNSWs is the responsibility of national coordinators, but the methods used should reflect the principles of the EMSWe CLD.

# Operational services and procedures

## Overview

This chapter provides a framework for the operational services and procedures to be maintained by both national and central components of the EMSWe CLD. For this chapter, operational services and procedures are understood as services and procedures that require human intervention.

MSs and the Commission/EMSA should take necessary steps to ensure that operators of the national and central components of this database are appropriately trained to perform their duties.

## Procedures for updating the EMSWe CLD

The Commission/EMSA must ensure that data available in the EMSWe CLD is up to date and of a good quality. This database needs to be synchronised with the UN/LOCODE list, the IMO GISIS Maritime security module and SSN to get the latest list of location records.

At the same time Member States must ensure that UN/LOCODES are identified and communicated to UNECE and that port facilities are created in the IMO GISIS Maritime Security Module.

The following procedures are proposed per type of code:

### UN/LOCODE

#### Updating UN/LOCODE list

As defined in Directive 2002/59/EC, Member States shall coordinate the creation, deletion and update of UN/ LOCODEs with the MS authorities that are officially designated to manage UN/LOCODEs (UN/LOCODE National Focal Points). The list of UN/LOCODE National Focal Points is available at:  http://www.unece.org/cefact/locode/focalpoint.html

The UN/LOCODE Entry Request System (https://apps.unece.org/unlocode/) enables registered users to submit request for new UN/LOCODE entries or for certain changes in existing UN/LOCODE entries. Registered users can submit request either one-by-one via a web-form, or by uploading a data file containing several requests, using a prescribed template.

Requestors will receive a response message, confirming the receipt of the data submitted and issuing a Request Reference Number for any subsequent communication with the secretariat. The response message will state whether the request has been Accepted, in which case it will be included in the next UN/LOCODE issue, or Noted, which means that further processing is needed but allowing the requestor to use his proposed code in the meantime. Rejected means that the name already figures in UN/LOCODE, or that the proposed code already is allocated; reasons for the rejection will be given.

#### Synchronising EMSWe CLD with UN/LOCODE

The synchronisation of the EMSWe CLD with the UN/LOCODE list will be done manually by the EMSWe CLD administrator (Commission/EMSA) until a system to system interface is established between the EMSWe CLD and the UN/LOCODE list. The UNECE Secretariat publishes a new UN/LOCODE list version twice a year (in July and in December). Once a new version is available, EMSWe CLD administrator shall download it and analyse its impact on the EMSWe CLD. The list of LOCODES to be added, updated and removed from the EMSWe CLD will be prepared and made available to national coordinators for the EMSWe for consultation.

National coordinators for the EMSWe will be requested to verify the list of changes and notify any issues detected. The absence of feedback will be considered as tacit agreement to the proposed changes. The changes to the EMSWe CLD will be implemented on the announced date. MNSWs shall be capable to process the changes received through the announcement service introduced in section 5 and update the location database at national level accordingly.

Proposal no 6: Considering that the EMSWe dataset requires port locations (function 1 - Port and function 7 - fixed platforms) but as well locations of other types (such as function 4 - airports), it is proposed to include UN/LOCODEs of all functions in the EMSWe CLD.

### Port facilities

#### Updating port facilities in IMO GISIS

As defined in the SOLAS Convention (Chapter XI-2 Regulation 13), the locations covered by the approved port facility security plans shall be registered by the Member States in the Maritime Security Module of the Global Integrated Shipping Information System (GISIS) which is maintained by the IMO. Member States shall coordinate the creation/deletion/updating of port facilities with the national authorities responsible for port facility security.

The national authority designated at IMO shall make changes via IMO GISIS web interface or if implemented at national level via SOAP based web services (established in line with the guidance for the electronic transfer of information to and from the Maritime Security module of GISIS).

#### Synchronising the EMSWe CLD with IMO GISIS

An automatic link between the EMSWe CLD and the IMO GISIS will be established in line with the guidance for the electronic transfer of information to and from the Maritime Security module of GISIS. This EMSWe CLD will request twice a day a list of changes in the IMO GISIS Maritime Security module and will reflect them accordingly in the database. MNSW shall be capable to process the changes received through the announcement service introduced section 5 and update the location database at national level accordingly.

### SSN specific codes

#### Updating SSN specific codes

According to the SSN LOCODE Guidelines, SSN specific code can be created by the EMSA’s Maritime Support Services (MSS) upon request from SSN NCA while the process of adding a new LOCODE to the UN/LOCODE list is in progress. Member States can request the creation of SSN specific code only for their own countries. The management of SSN Specific locations is carried out via the SSN Central Location Database.

Proposal no 7: It is proposed that SSN Specific codes can only be created by the EMSWe CLD administrator upon request coming from SSN NCA.

#### Synchronising EMSWe CLD with SSN specific codes list

The creation or update of SSN specific code will be available in the EMSWe CLD immediately after creation or update in the SSN Central Location Database. MNSW shall be capable to process the changes received through the announcement service introduced in section 5 and update the location database at national level accordingly

# Commissioning measures for establishing connection with MNSWs

## General guidance

Before connecting with the production site of the EMSWe CLD, a national coordinator for the EMSWe shall perform commissioning tests on the MNSW system and provide the data specified in the “MS Commissioning Test Plan” document, which is part of the technical and operational documentation, to the EMSWe CLD administrator. The commissioning tests verify that the system developed by a MS is able to exchange messages in accordance with the system specification.

The commissioning process is required to ensure that the MNSW can support the reliable, timely and accurate exchange of data and system information with EMSWe CLD. This process will cover all the EMSWe CLD messages transmitted to/from the EMSWe CLD system via the Web Services Interface. All test cases must be executed with fully automated system-to-system, in order to test the MNSW implementation.

Each Member State should have at least a test environment running in parallel with the production environment in order to test the interface with the EMSWe CLD Training environment. This test interface should allow the tester to run the tests whenever there is a new version of EMSWe CLD, or a new functionality impacting the data exchange. The Commissioning Plan document will be prepared to present the test cases and test scenarios to be used by Member States to support the CT process. The following test types will be covered by the CT:

- Functional Testing – the conformance of a MNSW application requires the correct functioning of all the features which have been converted into use cases.

- Data & Database Integrity Testing – The most crucial aspect of the application is that XML messages are correct, first in form, and then in content. The message validation, in accordance with XML standards and defined XML schema, is necessary. The Data Requester and the Data Provider need to follow the XML messaging specifications.

- Regression Testing – Regression testing is necessary to ensure that the previously executed tests are being re-executed against new versions of applications or deliverables. Regression testing ensures that the quality of the target has not been negatively affected when new capabilities have been added.

## Commissioning Procedure

Commissioning tests are performed at the request of MSs, and the results shall be documented in a test report. The test report, and the associated data files (if any), are to be submitted to the Commission/EMSA for assessment.

The Commission/EMSA shall analyse and evaluate the test report and, if the test results comply with the EMSWe requirements. Once the results have been validated, the Commission/EMSA issues a test acceptance form and updates the status of operation of the MS concerned. Via this process, the MS becomes an officially recognised participant in the EMSWe network for the designated functionality. The MNSW will be granted access to the EMSWe CLD Production environment following successful completion of the Commissioning Tests.

# System performance

The following performance requirements apply to the processing of messages and system information. Member State authorities may assign more specific performance standards in accordance with their national requirements.

## Timeframes for Data Availability

The MNSW systems and the EMSWe CLD should be supported by data communication links and networks that allow them to transfer information between the two systems within 1 minute. This timeframe should be respected for 95% of the information exchange over a 24h period, and for 99% over a one-year period.

## Timeframes for Data Storage

The active location codes shall be available via all interfaces in the EMSWe CLD without any time limitation. The information about deactivated location codes shall be available via all interfaces in the EMSWe CLD for a minimum of 5 years.

## System Availability Requirements

This section refers to the availability of the hardware and software necessary for the provision of the mandatory functionalities of the EMSWe CLD. It shall be maintained in operation twenty-four hours a day, seven days a week, to satisfy the mandatory system functionalities.

The availability of the system shall be maintained at a minimum of 99% over a period of one year, with the maximum permissible period of interruption of 12 hours. Scheduled interruptions of the service (e.g. planned maintenances) may exceptionally go over this limit. The availability requirements above apply also to each MNSW system as regards the communication links to the EMSWe CLD and to other national or local systems which they share location information with.

## Backup procedures

Backup procedures should be in place for each EMSWe CLD system component and should be activated in the event of a failure or a scheduled interruption. In the event of a failure or a scheduled interruption, the Commission/EMSA shall ensure that announcement messages are queued and then transmitted to the MNSW systems when communications and/or systems have recovered. The MNSW systems shall have capability to process this data.

The systems should be able to queue messages for at least 2 weeks. The body responsible for the affected system component must inform the other EMSWe participants according to the operational procedures whenever a failure or scheduled interruption occurs.

## Continuity of Service

To cover unforeseen crises, disasters and/or general disruptions to normal system operations, business continuity measures should be in place in order to ensure continuity of service at both national and central levels.

The business continuity measures should be defined at national and central level and aim at:

- Ensuring, by means of the alternative solutions, the availability of the location database to the MNSW;

- Ensuring, by means of the alternative solutions, the availability of the location database at national level through the MNSW;

- Ensuring that information is recoverable to the fullest extent possible after a downtime period/disaster/failure.

## Additional System Performance Requirements

Invalid messages will be rejected by the EMSWe CLD, and an error message will be sent back to the MNSW system. In cases where the EMSWe CLD transmits an invalid message, the MNSW should inform the Commission/EMSA of the reasons for the invalid message as soon as possible. All participants should aim to prevent invalid messages from being sent.

## Operational Coordination

Each national coordinator for the EMSWe and the Commission/EMSA should maintain a 24/7 contact point that is available to ensure the availability of the location database. The Commission/EMSA provides 24/7 monitoring of messages and network coordination as well as a helpdesk for the EMSWe CLD.

# System security

## General provisions

The following chapter describes the security policy that applies to the processing of messages and system information at the EMSWe CLD. The baseline security requirements hereunder should be mandatory for the EMSWe CLD and its interface with the MNSW systems and be referred to as optional at national/local level.

The national authorities may assign higher security measures on the system components they manage due to their specific needs and policies as long as these additional measures do not limit the ability of duly authorised MNSW users to access relevant information.

## Security measures

### Access Control

a. Authorities implementing the EMSWe CLD in form of hardware and software should keep record of individuals gaining physical access to it.

b. Tailoring of privileges granted to a MNSW user by the national coordinator for the EMSWe shall be performed as per access rights policy defined in section 5.

### Authentication

c. A reliable authentication mechanism shall be implemented to uniquely identify the EMSWe CLD users.

d. Passwords should be compliant with the SSN password policy detailed in the SSN Technical and Operational Documentation.

e. Authorities implementing a machine-to-machine interface shall guarantee the authenticity by appropriate means based on industrial best practices. For interfaces with the EMSWe CLD, 2-way SSL will be implemented.

f. A user ID should only be used by the appointed user or users. Authorities in charge of providing access to the location database should keep a record of all accesses per user ID at their system level.

g. A review of the credentials (e.g. password modification, user account revocation) used to access the EMSWe CLD should be performed at each system level regularly (at least each year) or whenever there is an upgrade of the security policy affecting the authentication mechanism (e.g. password policy).

### Authorisation

h. Authorisation of the national coordinator for the EMSWe by the Commission/EMSA, or of the authorities implementing a local system (e.g. port community system) by the national coordinator for the EMSWe should be subject to the identification of the individuals in their organization responsible for the security management and security implementation.

i. Authorities should grant access only to users in accordance with the rules in section 5.

j. A review of the authorisation/access rights should be performed at each system level at least each year or whenever there is an upgrade of the security policy affecting the authorisation protocol (e.g. change in the access right policy).

### Traceability and accountability

k. The EMSWe CLD allows the verification of the history, location, or application of the information from this database by means of documented recorded identification. National coordinators for the EMSWe are responsible for collecting this security data at national level.

l. The following actions shall be traced, and the records shall be available to the data provider of the information upon request:

- Receipt of the information.

- Modification of the information.

- Requests for the information.

m. The information recorded shall be as follows:

- User identification

- Time stamp.

- Description of action.

n. The EMSWe CLD and MNSWs shall ensure the non-repudiation and traceability of actions performed by users accessing the location database by means of both automated systems or the web interface (web-browser based mechanism). An administration providing the information can request the identity of the data requestor, without delaying the response.

### Confidentiality

o. Authorities implementing the EMSWe CLD system in form of hardware and software and/or with responsibilities in terms of provision of access rights shall ensure that the confidentiality of the data stored within or exchanged by those system components they are responsible for is not compromised. Data protection procedures shall be put in place.

p. Data storage shall be performed in accordance with the following rules:

- System Security Related data shall be stored encrypted;

- Commercial Sensitive data shall be protected according to the access rights policy.

- Personal data should be stored in accordance with point (t) below.

r. Users shall not provide information to any unauthorised persons or systems.

s. Users shall not disclose their login credentials to unauthorised persons.

t. The principles of personal data protection shall be applicable to any information concerning an identified or identifiable person exchanged through SSN system.

### Integrity

u. The EMSWe CLD shall ensure that the information is authentic and complete. The information transmitted to MNSWs shall only be modified by the administrator.

v. The management of data provision shall ensure all reasonable steps have been taken to prevent denial of service attacks, the introduction of ‘malware’, or other malicious events with the potential of compromising EMSWe CLD functionalities.

w. The list of hardware and software, used to implement the EMSWe CLD or used within the authority to interface with it, should be recorded in a register. This register should be maintained during the whole system lifecycle.

### Training

x. Authorities should ensure that all system users within its jurisdiction are aware of the security requirements of the system and have the knowledge and competencies to fully discharge their obligations.

### Audit

y. Security audits on the EMSWe CLD implementation and usage should be carried out on a regular basis. Whenever possible existing audit arrangement will be accepted as the fulfilment of this requirement.

x. Following a security breach there should be an investigation to identify the cause and any remedial actions required.

Data elements of the EMSWe CLD

[See attached XLSX file]

Designated Authority means the organisation(s) or the administration(s) identified, within the Contracting Government, as responsible for ensuring the implementation of the provisions of this chapter pertaining to port facility security and ship/port interface, from the point of view of the port facility (source: SOLAS, chapter XI-2, Regulation 1).

Ship/port interface means the interactions that occur when a ship is directly and immediately affected by actions involving the movement of persons, goods or the provisions of port services to or from the ship.