Guidance for the operational tests of the Integrated Report Distribution (IRD) service

Facilitation of ship to shore reporting pilot project

Version 0.1

Date: 17/03/2021

Document History (if needed)

| Version | Date | Changes | Prepared |
| --- | --- | --- | --- |
| 0.1 | 17/03/2021 | Draft | EMSA |

Table of Contents

1.	Background	2

2.	Objective of the document	2

3.	Integrated Report Distribution	2

3.1	What is the IRD?	2

3.2	What data is available?	2

3.3	How can I access this information?	3

3.1	What are the available triggers for generating ISR?	3

4.	Who can participate in tests?	4

5.	Possible use cases for operational tests	4

5.1	Use of IRD in MRS and VTS areas	4

5.2	Use of IRD to increase awareness in Traffic Separation Schemes without SRS	4

5.3	Use of IRD in MRCC or border control centres	5

5.4	Use of IRD to monitor specific ships regardless of their area of operations	5

6.	How to express interest in participating?	5

7.	Planning	5

8.	Helpdesk	5

9.	How to report test results?	5

Annex 1 – Registration Form	6

Annex 2 – Test Report	8

# Background

On 21 November 2018, EMSA sent a letter to the directors of maritime administrations of the EU Member States and of EEA Countries to participate in a pilot project for the facilitation of ship to shore reporting. This pilot project aims at developing and testing solutions to facilitate the reporting procedures from ship to shore and improve the coastal stations’ situation awareness.

The project concept foresees testing of technical solutions with Member States’ authorities and industry, such as the Integrated Report Distribution (IRD) service. EMSA should prepare with the Member States guidance documentation for the operational testing of the IRD service.

# Objective of the document

The objective of this document is to provide guidance to MS authorities willing to participate in the operational tests of the facilitation of ship to shore reporting pilot project.

These operational tests will be used to verify the quality, availability and reliability of the IRD service developed under the project and to improve or adjust the solution.

These operational tests will be used to verify quality, availability and reliability of the technical solution developed within the project and to improve or adjust the solution.

This guidance document is applicable to IRD phase 2.

# Integrated Report Distribution

## What is the IRD?

For the purpose of the facilitation of ship to shore reporting pilot project, a new “Integrated Reports Distribution” (IRD) service has been developed. The IRD service uses existing back-end services from the SSN Ecosystem to prepare and distribute to Member States’ coastal stations reports on ships sailing in their areas of interest.

The coastal stations authorities can use the Integrated Ship Reports (ISR) provided by the IRD service to minimise the reporting burden for ships, to improve the efficiency of provided services and to develop a better awareness of their areas of control.

## What data is available?

The ISR report presents consolidated information on an individual ship. It is made of data elements which can be configured by the end user allowing the creation of tailor-made report for different costal stations. The following data elements may be included the ISR:

- SafeSeaNet European Index Server (EIS) data:

- Current ship’s voyage data including:

  - Port call information – acquired from pre-arrival notifications sent 72 and 24 hours before arrival and information on ship arrivals and departures;

  - Summary of hazmat information – acquired from pre-arrival and departure notifications on the carriage of dangerous and marine polluting goods;

  - Summary of security information – acquired from pre-arrival notifications on security information;

  - Summary of waste information – acquired from pre-arrival notifications on waste and cargo residues;

  - Availability of bunkers information – indication if information on bunkers carried onboard was reported;

- Incident information – Information on accidents and incidents which have occurred at sea;

- MRS information – reports received in the IMO adopted Mandatory Reporting Systems;

- MRS information – reports received in the IMO adopted MRS systems;

- Information on exemptions – information regarding exemptions granted to the ship.

- Vessel identifiers from ship database:

The main vessel identifiers such as IMO number, MMSI number, Ship Name, Call Sing, Flag and Ship Type retrieved from EMSA’s ship database.

- Ship Tracking Information

The latest ship position coming from T-AIS and S-AIS systems.

- Vessel voyage plan from the Voyage Information Service (VIS)

One of the services developed by the STM project is the Voyage Information Service (VIS) which aims at facilitating the sharing of vessels’ voyage plans with shore-based service providers such as coastal stations. A voyage plan including waypoints and estimated time of arrival to these waypoints will be made available in the ISR. At the date of writing of this document, around 20 ships participating in the VIS service make voyage plans available to IRD.

## How can I access this information?

The Integrated Ship Report (ISR) is available through a web user interface (via EMSA’s portal), system to system interface (SOAP and REST protocols) and e-mail.

In order to access data via system to system interface the national/local system needs to be upgraded.

## What are the available triggers for generating ISR?

The IRD system monitors ships sailing in areas of interest defined by the authority and automatically sends an ISR when a specific event occurs (e.g. entry of a ship in the area, exit of a ship from the area, Incident Report received for a ship in the area, etc.). The configuration of the area and the events is done by EMSA following request received from the Member State authority.

In addition, it is also possible to create list of ships of interest that will be monitored by the IRD and for which ISR will be generated when specific event occurs, e.g. entry to a port, Incident Report received, at specific time interval (e.g. once a day), etc..

It is also possible to manually request the generation of an ISR for a specific ship. This can be done by the user in the web user interface or via system to system interface by providing one of the ship identifiers.

# Who can participate in tests?

There are 14 Member States participating in the pilot project: BE, DE, DK, EE, FI, FR, HR, IT, LV, NL, NO, PL, RO and SE.

The project was initially established with the objective to facilitate the work of coastal stations’ operators in MRS and VTS areas. During the execution of the project it was found that the deliverables of the project would as well be beneficial to other authorities executing function in the maritime domains, e.g. SAR, MRCC, border control, port authorities, PSC, etc.

The Member State representatives participating in the project will designate the authorities that can participate in the testing of the IRD.

All participants need to have an EMSA Portal account. The end-users will only have access to the information they are entitled to in accordance with the access rights assigned by the SSN administrators for EIS data and by the IMS administrators for the ship tracking information.

Access to vessels’ voyage plans can be given to all project’s participants.

# Possible use cases for operational tests

The use cases presented in this section have been identified by EMSA and Member States participating in the pilot project. They are only indicative. The authorities willing to participate in the tests can either customise them or even use the system for other operational purposes.

The purpose of the tests is to verify the quality, reliability and correctness of the information provided in the ISR reports.

## Use of IRD in MRS and VTS areas

There are 16 IMO adopted mandatory ship reporting systems (MRSs) and around 100 Vessel Traffic Services (VTSs) in the European waters where ships are required to report data to shore-based authorities.

The coastal stations responsible for MRS/VTS may use the relevant information from the ISR to cross-check it with the MRS/VTS report provided by a ship

There are two possible ways of testing the IRD in MRS/VTS areas. The IRD system can be either configured to generate a report automatically upon the ship’s entry to an area, or an operator may manually request the report it at any time.

The expected benefits of the IRD service in the future could be:

- MRS/VTS operators should not need retyping information which would be already available and pre-filled in their system before the vessel reaches the reporting point;

- Ships should only be requested to confirm information already available (rather than having to report all information again).

## Use of IRD to increase awareness in Traffic Separation Schemes without SRS

There are many Traffic Separation Schemes (TSS) in EU waters when no reporting from ships is required. In many cases these coastal stations covering those TSS do not have access to SSN data, or it is too complex to collect necessary data in short period of time. The IRD service can be used to provide integrated report about ships passing through the TSS areas.

There are two possible ways of testing the IRD service in these areas. The IRD system can be either configured to generate a report automatically upon a ship’s entry in the TSS area or an operator may manually request the report at any time.

## Use of IRD in MRCC or border control centres

Authorities such as MRCC or border control centres may use the IRD service to have better awareness about ships in their respective areas of responsibility. Those authorities may decide to get an ISR for all ships getting into an area or get ISR for specific ships of interest based on the following criteria’s: ship flag, ship type, ship IMO, ship MMSI. For the ships being in the area of responsibility, it is possible to get an ISR upon following triggers: Incident report received, MRS report received, Hazmat received, Security Level higher than 1, Arrival at port.

Apart from receiving ISR reports in an automatic way these authorities may also use manual request whenever needed.

## Use of IRD to monitor specific ships regardless of their area of operations

The IRD service may be also used to generate ISRs for the ships regardless of their area of operation. As an example, an authority may be interested in receiving an update at specified time intervals (e.g. daily) for a group of ships of their interest. This feature can be used for example by Flag States to get update on their fleet. Apart from update at specific time intervals, it is also possible to get a report when certain event occurs (e.g. if an Incident Report is received for a ship being monitored).

# How to express interest in participating?

In order to participate in the operational tests, Member State representatives shall fill in the form available in Annex 1 and send it via e-mail to EMSA (irdpilot@emsa.europa.eu). One form per authority shall be provided.

Webinar training sessions will be organised for the authorities involved in the tests.

# Planning

The tests will run in the period between April and June 2021.

# Helpdesk

Problems related to connection issues, technical or functional difficulties or other issues that may occur during the operational tests can be reported to:

E-mail: irdpilot@emsa.europa.eu

Helpdesk service will be provided on workdays (Monday to Friday out of bank holidays) and on working hours in Lisbon, Portugal.

# How to report test results?

The authority participating in the operational test shall prepare a test report using the form available in Annex 2. This report shall be sent as a Microsoft Word file or PDF file to EMSA (lirdpilot@emsa.europa.eu) either by the authority or by Member State representative.

One test report per IRD distribution service shall be provided.

### Annex 1 – Registration Form

This form shall be provided per participating authority. One or several person users may be indicated in the form.

| Country |  |
| --- | --- |
| Authority Name |  |
| Type of authority | ☐ VTS centre ☐ SAR ☐ MRCC ☐ Border control ☐ Port authority ☐ Flag State ☐ PSC ☐ Other (please specify): |
| Short description of foreseen operational tests |  |
| Point of contact (please specify one PoC for operational tests) | Name: E-mail: Phone: |
| ISR distribution method | ☐ User web interface ☐ E-mail ☐ System to system communication |
| Trigger for ISR (More than one option can be selected.) | Option 1: ☐ Entry/exit to/from an area ☐ Specify the area of interest by providing the coordinates of its polygon: ☐ All ships (if you want to monitor all available ships) ☐ Specific ships: ☐ List of ships (please specify the IMO numbers or MMSI numbers): ☐ Ships of specific flag (please specify the flag(s)): ☐ Ships of specific type (please specify the ship type(s)): Option 2: ☐ Monitoring of specific ships regardless of the area of operation (please specify the ships’ IMO number or MMSI numbers): ☐ ISR generated at specific time interval (specify how often you want to receive update (e.g. every 1 day): ☐ ISR generated upon certain event: ☐ Incident Report received ☐ MRS report received ☐ Security level > 1 ☐ Indication of Hazmat on board received Option 3: ☐ Ad-hoc manual request to be made in the web user interface |
| Blocks of data to be included in the ISR | ☒ Header (mandatory block) ☒ Vessel identification (mandatory block) ☐ Position report (optional) ☐ Voyage data (optional) ☐ Latest Incident Report (optional) ☐ Latest MRS report (optional) ☐ Active Exemptions (optional) ☐ Last 5 MRS reports received (optional) ☐ Incident Reports received in the last 12 months (optional) ☐ Voyage Plan (optional) |
| Users for which access shall be granted to IRD service (only if distribution method user web interface selected) | Specify the EMSA user IDs: |
| E-mail addresses for receiving ISR reports (only if distribution of ISR by e-mail selected) | Specify the email addresses: |
| Planned dates for operational tests |  |

### Annex 2 – Test Report

| Country |  |
| --- | --- |
| Authority Name |  |
| Name of the distribution service from the IRD service |  |
| Dates when the tests were executed |  |
| General feedback |  |
| Approximate number of ISRs validated during the tests |  |
| Detailed feedback per ISR attribute or block of data This feedback can be provided as: - percentage (e.g. 80% of the MMSI provided in the ISR were correct - number or 25 out of 30 (83.3%) of the MMSI provided in the ISR were correct - descriptive text (e.g. most of the time correctly provided, most of the time not available, most of the time incorrect, always available and correct, etc.) | Block of data; Feedback from MS. Vessel Identification; Please provide detailed feedback about this block of data. IMO. MMSI. Call Sign. Ship Name. IR Number. Flag. Ship type. Position Report; Please provide detailed feedback about this block of data. Source. Originator. Data and time. Latitude. Longitude. True Heading. Heading. Speed. Navigational status. Length. Breadth. Voyage data; Please provide detailed feedback about this block of data. ShipCallIDDeparture. ShipCallIDArrival. Purpose of Call. Brief cargo description. Previous Port of Call. ATD from Previous Port of Call. ETD from Previous Port of Call. Port of Call. Position in Port of Call. ETA to Port of Call. ATA to Port of Call. ETD from Port of Call. ATD from Port of Call. Number of Persons on board. Hazmat on board. Waste delivery status. Current Ship Security level. Bunkers details provided. Subsequent Port of Call. ETA to Subsequent Port of Call. Latest Incident Report; Please provide detailed feedback about this block of data. Date and time of IR. Type of IR. IncidentID. Latest MRS report; Please provide detailed feedback about this block of data. Date and time of MRS report. MRS identification. Port of Call. ETA to Port of Call. Number of Persons on board. Latitude. Longitude. Hazmat on board. Active Exemptions; Please provide detailed feedback about this block of data. Last 5 MRS reports; Please provide detailed feedback about this block of data. Incident Reports last 12 months; Please provide detailed feedback about this block of data. Voyage Plan; Please provide detailed feedback about this block of data. |
| Availability of the IRD service Please specify if there were any downtimes of the system during your tests |  |
| Performance of the IRD service Please specify if the system was responding to your requests in a timely manner |  |
| Feedback on support received from EMSA |  |
| Ideas for improvements |  |
| Would you like the IRD service to become one of EMSA’s permanent maritime services following the pilot project? |  |

The pilot project is executed under the EMSA “Interoperability Project” which was set up to execute action 1.2.1.4 of the EMFF work programme 2018 for “Promotion of interoperability between industry and competent authorities in the European Maritime Single Window (EMSW) environment under the CISE Process”.

Voyages are acquired by consolidating information from PortPlus notification provided by the last port (port of departure) with information from PortPlus notification provided by the port of arrival. A voyage starts at the departure from last port and lasts until the departure from the port of arrival.

The STM Validation Project is a continuation of the EU funded MONALISA and MONALISA 2 and aim at demonstrating the STM concept in large-scale test beds in both the Nordic and Mediterranean Seas.